WinSTBUpgrader User Guide

Revision Number: 0.0

Revision Date: 2018/09/19

Montage Technology Proprietary and Confidential

Revision History

| Revision Number | Revision Date | Changes |  |
| --- | --- | --- | --- |
|  |  | Page Number | Description |
| 0.0 | July 30, 2026 | - | Initial Release |

Table of Contents

1、What is WinSTBUpgrader	3

2、Functional Module	3

3、How to use software	4

4、btinit and client	5

# 1、What is WinSTBUpgrader

The main functions of this tool are serial port burning flash, usb burning flash, otp reading and writing and verify；

# 2、Functional Module

- Upgrade： serial port burning flash；

- Dump：    read flash data；

- Download：upgrade flash by usb；

- Verify：    Write protection module；

- OTP：     Otp data read and write module；

# 3、How to use software

Upgrade、Dump、OTP has Same process Here is an example of Upgrade：

Label 1: Open the serial port；

Label 2: Select the desired functionality；

Label 3: Select the appropriate platform；

Label 4: Choose normal or secret；

Label 5: Select whether to erase；

Label 6: Fill in the key values required for syn, for example, conax presses the ‘M’ key（Only secret         is visible）

Label 7: With label6 visible，Click Start and power up STB to finish. 5 seconds later, click Stop, otherwise the synchronization will fail.

Label 8: Select the load file path (dump is the save path);

Label 9: Click Start to send btinit and client, and starts the process starts after success.

Download The steps are as follows：

Name the falsh file flash.bin and store it in the U disk.

Label:1：Select the flash type.

Label 2：Select the linux_upg_config.xml upgrade configuration file.

Ps：The burn address can be modified in the linux_upg_config.xml file.

Label 3：After clicking Start, wait for the board to Start printing successfully.

2. CMD command burning Symphony

Before using this command, make sure that the STB is in CMD > mode, otherwise you need to restart the STB manually.

- C: Serial slogans must be capitalized

- P: Symphony only

-D：choice DDR type DDR2 or DDR3 must be capitalized

- U: Absolute path for burning files under Windows, please note the use of "\\"

- Q: Whether to exit the tool automatically after burning success

Example:

WinSTBUpgrader.exe-C COM1-P Symphony –D DDR2

-U C: Users Jiyzhang. MONTAGE-TECH Desktop flash_ddr2_400m. bin-Q

Use COM1 to burn flash_ddr2_400m. bin to STB through Symphony ddr2, and close the tool after completion.

Note: Please note the order of commands and case parameters. Case switching is not supported for the time being.

3、Symphony2 Nagra Burn

Label a: Open the serial port;

Label b: Add a burn task;

Label c：Delete a burn task;

Label d：Click to start burn process;

Label e：Import history burning task from file;

Label f：Export current burning task to file;

Label g：Fill in the path of the file to be burned;

Label h:  Click to select the file you want to burn;

Label i：Fill in the start address of th current burn file;

Label j：Fill in the size of the current burnt file (must be greater than or equal to the current file size,       otherwise the default file size);

Use steps:

1. Click Label B to add blank tasks to be burned.

2. Set up the list to be burned through Label g, h, I and J.

3. Click Label a to open the serial port.

4. Click Label d to start burn.

Before using, please fill in the list of tasks to be burned in order to ensure that the serial port is correct and open. Click Start to start burning.

# 4、btinit and client

The customer can replace different btinit and client;

| Platform/function | Btinit | Client |
| --- | --- | --- |
| Symphony normal upgrade、dump | btinit_ddr2_400m_debug.img btinit_ddr3_666m_debug.img | upgclient_symphony.bin |
| Symphony secret upgrade、dump | btinit_ddr2_secure.img btinit_ddr3_secure.img | upgclient_symphony.bin |
| Symphony normal OTP | btinit_ddr2_400m_debug.img btinit_ddr3_666m_debug.img | upgclient_symphony_otp.bin |
| Symphony secret OTP | btinit_ddr2_secure.img btinit_ddr3_secure.img | upgclient_symphony_otp.bin |
| Symphony normal Verify | btinit_ddr2_400m_debug.img btinit_ddr3_666m_debug.img | symphony_flash_protect_test.bin |
| Symphony secret Verify | btinit_ddr2_secure.img btinit_ddr3_secure.img | symphony_flash_protect_test.bin |
| Symphony normal Download | btinit_ddr2_400m_debug.img btinit_ddr3_666m_debug.img | ub_sym.img |
| Symphony secret Download | btinit_ddr2_secure.img btinit_ddr3_secure.img | ub_sym.img |
| Symphony2 upgrade、dump | btinit_ddr2_symphony2_debug.img btinit_ddr3_symphony2_debug.img | upgclient_symphony2.bin |
| Symphony2 OTP | btinit_ddr2_symphony2_debug.img btinit_ddr3_symphony2_debug.img | upgclient_symphony2_otp.bin |
| Symphony2 Verify | btinit_ddr2_symphony2_debug.img btinit_ddr3_symphony2_debug.img | symphony2_flash_protect_test.bin |
| Symphony2 Download | btinit_ddr2_symphony2_debug.img btinit_ddr3_symphony2_debug.img | ub_sym.img |